# Keldysh Input Parameters of ZnSe from Keldsyh_II_various.py

Independent NIR and LWIR input specification; assigned, computed, and missing fields are distinguished explicitly

# 1. Formula and Code Convention

γ = (ω/e) √[(mᵣ e c n ε₀ E_g)/I]

The script follows the source-paper convention E = √[I/(cnε₀)], so γ contains I, not 2I, in the denominator. E_g is entered in eV and SI units are used internally. The code now uses exact SI definitions and 2022 CODATA values: c = 299792458 m/s, ε₀ = 8.8541878188×10⁻¹² F/m, e = 1.602176634×10⁻¹⁹ C, ℏ = 1.054571817×10⁻³⁴ J·s, and m₀ = 9.1093837139×10⁻³¹ kg.

# 2. Material and Laser Parameters: Assignment versus Calculation

| Code field / symbol | Meaning | Value or logic used for ZnSe | Unit | Status in various code |
| --- | --- | --- | --- | --- |
| mat_flag | Material selector | 1 | Dimensionless | Hard-coded case assignment. |
| E_g | Bandgap energy | 2.7 | eV | Hard-coded in material_flag1(). |
| mᵣ | Electron–hole reduced effective mass | 0.17 m₀ | kg after ×m₀ | Hard-coded ratio to m₀. |
| n₀ | Linear refractive index | n₀ = √(n²); wavelength dependent | Dimensionless | Computed from the coded dispersion expression. |
| n₂ | Nonlinear refractive index | 2.3×10⁻¹⁸ (λ<2); 6.5×10⁻¹⁹ (λ≥2) | m²/W | Piecewise hard-coded assignment. |
| R | Reflectance used by the code | 0.32949 (λ<2); 0.31059 (λ≥2) | Dimensionless | Piecewise hard-coded assignment; not computed from n₀. |
| T | Interface transmission factor | T = 1 − R | Dimensionless | Computed. |
| λ | Vacuum wavelength | 0.8 (NIR); 9.2 (LWIR) | µm | Hard-coded case assignment. |
| τ | Intensity-FWHM pulse duration | 100 (NIR); 2000 (LWIR) | fs | Hard-coded case assignment. |
| F₀ | Peak fluence | 0.112 (NIR); 0.83 (LWIR) | J/cm² | Hard-coded case assignment. |
| a | 1/e² intensity radius along first principal axis |  | µm | Not present in Keldsyh_II_various.py; left blank. If later supplied, use a 1/e² intensity radius (half of the full 1/e² intensity diameter). |
| b | 1/e² intensity radius along second principal axis |  | µm | Not present in Keldsyh_II_various.py; left blank. If later supplied, use a 1/e² intensity radius (half of the full 1/e² intensity diameter). |
| I₀ | Gaussian temporal peak intensity | From F₀ and τ | W/cm² | Computed; no beam radii enter this conversion. |
| γ | Material-specific Keldysh parameter | From λ, I₀, n₀, E_g, and mᵣ | Dimensionless | Computed. |

# 3. ZnSe Refractive-Index and Optical-Branch Logic

For mat_flag = 1, material_flag1() calculates n₀ = √(n²) from the following wavelength-dependent expression, with λ in µm:

n² = 1 + 4.45813734λ²/(λ²−0.200859853²) + 0.467216334λ²/(λ²−0.391371166²) + 2.89566290λ²/(λ²−47.1362108²)

For λ < 2 µm, n₂ = 2.3×10⁻¹⁸ m²/W and R = 0.32949; for λ ≥ 2 µm, n₂ = 6.5×10⁻¹⁹ m²/W and R = 0.31059. The script then computes the interface transmission factor as T = 1 − R.

# 4. Enabled NIR and LWIR Cases

| Case input | Code value | Status / note |
| --- | --- | --- |
| NIR wavelength λ | 0.8 µm | Assigned in get_cases(). |
| NIR duration τ | 100 fs | Assigned intensity FWHM. |
| NIR fluence F₀ | 0.112 J/cm² | Assigned peak fluence. |
| NIR beam radius a |  | Not present; left blank. Any future value must use the 1/e² intensity-radius convention. |
| NIR beam radius b |  | Not present; left blank. Any future value must use the 1/e² intensity-radius convention. |
| LWIR wavelength λ | 9.2 µm | Assigned in get_cases(). |
| LWIR duration τ | 2000 fs | Assigned intensity FWHM. |
| LWIR fluence F₀ | 0.83 J/cm² | Assigned peak fluence. |
| LWIR beam radius a |  | Not present; left blank. Any future value must use the 1/e² intensity-radius convention. |
| LWIR beam radius b |  | Not present; left blank. Any future value must use the 1/e² intensity-radius convention. |

Peak intensity is computed from the Gaussian-pulse relation I₀ = (2F₀/τ)√[ln(2)/π]. The case-specific γ values below are then computed with this material's n₀, E_g, and mᵣ.

# 5. Derived Case Values

| Condition | n₀ / n₂ (m²/W) | R / T | I₀ (W/cm²) / γ |
| --- | --- | --- | --- |
| NIR: 0.8 µm, 100 fs | 2.52418 / 2.3×10⁻¹⁸ | 0.32949 / 0.67051 | 1.05217×10¹² / 3.03534 |
| LWIR: 9.2 µm, 2000 fs | 2.41115 / 6.5×10⁻¹⁹ | 0.31059 / 0.68941 | 3.89866×10¹¹ / 0.423787 |
| Assignment status | n₀ computed; n₂ assigned | R assigned; T computed | I₀ and γ computed |

The reported n₀, T, I₀, and γ values are derived outputs, not independent input assignments. Values are rounded for documentation; the script uses its full floating-point results.

# 6. Missing Inputs and Model Scope

| Requested / related field | Value in various code | Interpretation |
| --- | --- | --- |
| Binding energy ε_b |  | Not present and not used by the Keldysh/avalanche workflow. |
| Valence-band density |  | Not present and not used as a model input. |
| Beam radii a and b |  | Not present; fluence is supplied directly, so the fields remain blank. Any future radii must use the 1/e² intensity convention. |
| Propagation and losses | Not modeled | The zero-D workflow neglects propagation, depletion, saturation, recombination, and trapping. |

# 7. Source and Code Traceability

• Source script: Keldsyh_II_various.py, material_flag1(), lines 121–202.

• Enabled cases: get_cases(), lines 209–260.

• Fluence-to-peak-intensity relation: peak_intensity_from_fluence_wm2(), beginning at line 267.

• Electric-field and Keldysh-rate convention: keldysh_full_rate_m3_s(), beginning at line 413.

• The comparison plot's secondary γ axis uses ZnS as a common reference, but each material's Keldysh photoionization rate uses its own n₀, E_g, and mᵣ.

• The source script contains no literature citations for these hard-coded material values; this document records code provenance and does not upgrade an assignment into a verified material constant.

Concluding input note: this specification reproduces the ZnSe values and logic presently used by Keldsyh_II_various.py. Verify the hard-coded band gap, reduced mass, dispersion, n₂, reflectance, and experimental fluences before treating the results as publication-grade material predictions.